Bangladesh University of Engineering and Technology

CSE 306

Computer Architecture Sessional

Assignment-3: 4-bit MIPS Design, Simulation, and Implementation

Section - B1

Group - 03

Members of the Group:

1. 2005071 – Sagor Chanda

2. 2005069 – Souvik Mandol

3. 2005076 – S. M. Kausar Parvej

4. 2005068 – Md. Sumon Hossain

5. 1705083 – Hozifa Rahman Hamim

- Introduction

MIPS (Microprocessor without Interlocked Pipeline Stages) is a RISC (Reduced Instruction Set Computer) architecture widely used in embedded systems, networking devices, and other applications requiring efficient processing power. Its fixed instruction set ensures regularity and simplicity in instruction execution.

Here is an example of add instruction:

| Operation | Example | Instruction | Corresponding Action |
| --- | --- | --- | --- |
| Addition | x = a + b | add $t2, $t1, $t3 | $t2 = $t1 + $t3 |

In MIPS, instructions are typically executed in a pipeline fashion, where each stage of the pipeline handles a different aspect of instruction execution, such as instruction fetch, decode, execute, memory access, and write back. This pipelining allows for efficient processing of multiple instructions simultaneously.

The MIPS datapath consists of various components, including registers, ALUs (Arithmetic Logic Units), MUXs (Multiplexers), memories, and control elements, which work together to process instructions efficiently. The datapath is responsible for executing instructions by performing operations such as arithmetic and logic operations, memory accesses, and control transfers.

- Instruction Set

In this assignment we are designing a 4-bit MIPS which is consists of:

- 8-bits Address bus

- 4-bits Data bus

- 4-bit ALU

- Temporary registers are: $zero, $t0, $t1, $t2, $t3, $t4

Instruction set for this MIPS is given below:

Table 1: Instruction Set Description

Our MIPS Instructions will be 16-bits long with the following four formats

Table 2: Instruction Format

Opcodes of the instructions are of 4 bits, so between 0 and 15. We’re given the Instruction assignment POJMBNDECAGIHKLF. So, our instruction set would be something like this:

| Opcode | Instruction ID | Category | Type | Instruction |
| --- | --- | --- | --- | --- |
| 0000 | P | Control-unconditional | J | j |
| 0001 | O | Control-conditional | I | bneq |
| 0010 | J | Logic | S | srl |
| 0011 | M | Memory | I | lw |
| 0100 | B | Arithmetic | I | addi |
| 0101 | N | Control-conditional | I | beq |
| 0110 | D | Arithmetic | I | subi |
| 0111 | E | Logic | R | and |
| 1000 | C | Arithmetic | R | sub |
| 1001 | A | Arithmetic | R | add |
| 1010 | G | Logic | R | or |
| 1011 | I | Logic | S | sll |
| 1100 | H | Logic | I | ori |
| 1101 | K | Logic | R | nor |
| 1110 | L | Memory | I | sw |
| 1111 | F | Logic | I | andi |

Table 3: Instruction for section B1-Group 3

- Complete Block diagram of a 4-bit MIPS processor

- Block diagrams of the main components

- Approach to implement the push and pop instructions

- ICs used with their count

| IC Number | IC Name | Count |
| --- | --- | --- |
| ATmega 32 | ATmega | 6 |
| IC 74157 | Mux | 6 |
| IC 7483 | Adder | 2 |

- Discussion

- For implementing the circuit in software level , we used 7400-library integrated circuits. In hardware level, necessary coding has been done at ATMEGA level.

- In hardware level, we showed the value of each register by setting ShowReg in the register file On. From the ALU, we demonstrated output in each step of operation.

- The value of PC was also shown for each clock cycle. The value of each data memory was also visible at software level.

- All of our ATmega were tested before integrating it to our final circuit.

- Wires with different sizes were used to make the whole circuit organized and easily understandable for the user.

- The outputs were checked multiple times for a large number of input test cases to ensure that our circuit satisfies the expected output values from the given input file.

- In order to optimize the number of IC , we performed PC+1 using the Cin value of the adder. Similarly instead of generating a circuit to create a selector bit bsel ( whether to branch or not ) , we calculated the value in ATmega designated for ALU operation .

- Clock pulse was provided using a Push Button Switch.

- Contribution

| ID | Contribution |
| --- | --- |
| 2005076 | Software implementation, Hardware implementation, Debugging, Report writing |
| 2005071 | Hardware implementation, Report writing, Debugging |
| 2005069 | Hardware implementation, Report writing, Debugging |
| 2005068 | Hardware implementation, Report writing, Diagram drawing |
| 1705083 | Hardware implementation, Report writing, Debugging |